	[image: image2.wmf]FUGRO SPATIAL SOLUTIONS PTY LTD


Metadata Report 
	[image: image2.wmf]



	Record ID 
	Record 

	Metadata Filename 
	22496201_MMG_Tasmania

	Metadata Language 
	English 

	Metadata Character Set
	ASCII (utf8)

	Metadata Point of Contact 
	Tony McDade [Project Manager] 

Fugro Spatial Solutions Pty Ltd 

Level 1, Brandl Street, 

Eight Mile Plains Qld 4113

Telephone: +61738413433 

T.McDade@fugro.com

	Metadata Date Stamp 
	26/02/2014

	Dataset Title and Graphic Overview
	MMG ALS ADS Tasmania


	Dataset Reference Date
	Feb-2014

	Dataset Description Abstract
	MMG LiDAR derived products 

	Dataset Language 
	English 

	Dataset Character Set
	ASCII

	Dataset Topic Category
	ALS DEM and Contours

	Geographic Location Description
	Rosebery, Tasmania, Australia
[image: image1.png]1159 m

1,000m

750m

500m

250m

97m

0.0km







	Geographic Bounding Box
	WEST LONGITUDE=145° 19' 55" E

NORTH LATITUDE=41° 43' 6" S

EAST LONGITUDE=145° 38' 8" E

SOUTH LATITUDE=41° 55' 52" S

	Acquisition Start Date
	09/01/2014 [UTC]  

	Acquisition End Date
	12/01/2014 [UTC]  

	Sensor Type Name 
	Leica ALS50-2

	Flying Height (AGL) metres
	1051-2067m

	INS / IMU Used 
	IPAS10

	Number of Runs 
	52

	Swath Width (metres)
	990

	Flight Direction 
	Non-Cardinal 

	Side Overlap (%) 
	23

	Output Data Format 
	· Lidar Ground points in ASCII XYZ format

· Lidar Tops of trees/veg. in ASCII XYZ format

· Contours 0.5m in DXF and DGN formats
· Ground gridded DEM 2m & 1m tiles in ASCII XYZ format and overall ESRI ASCII Grid
· Intensity Images are in Geotiff tiles and Overall ECW format

· Lidar Classified data in LAS format

· LiDAR Ground Model Key Points in ASCII XYZ format



	Horizontal Datum 
	GDA94

	Vertical Datum 
	AHD

	Map Projection 
	MGA Zone 55 

	Number of control points

DZ Validation 
	27 inside the survey area
East

Number               Easting     Northing   Known Z   Laser Z        Dz

-----------------------------------------------------------------------

XP04B             382828.700  5373526.730   479.970   480.070    +0.100

PID02SQUARE       382522.610  5365465.610   853.030   852.960    -0.070

PID01NAIL         376001.560  5365064.490   932.490   932.430    -0.060

XP15B             385443.600  5380962.520   187.370   187.420    +0.050

XP03B             375412.520  5372941.980   154.680   154.730    +0.050

XP16C             379643.330  5362477.160   525.170   525.210    +0.040

XP03              375408.700  5372952.310   153.980   154.020    +0.040

XP06C             376001.710  5365066.830   932.520   932.480    -0.040

XP19B             385009.530  5375187.510   174.740   174.770    +0.030

PID01B            376003.380  5365066.520   932.590   932.560    -0.030

XP06B             378982.190  5366465.760  1118.190  1118.160    -0.030

XP16D             379646.430  5362477.230   525.170   525.190    +0.020

XP04              382828.780  5373522.450   479.950   479.970    +0.020

XP02B             378882.340  5374046.060   185.360   185.340    -0.020

XP14B             377747.930  5378928.750   163.990   163.970    -0.020

XP14              377746.840  5378933.100   164.040   164.020    -0.020

XP15              385415.630  5381027.370   190.860   190.880    +0.020

PID02CROSS        382516.340  5365461.630   851.520   851.510    -0.010

XP06A             378966.900  5366432.710  1119.190  1119.200    +0.010

XP19              384989.910  5375184.040   174.890   174.880    -0.010

XP06D             376055.440  5365138.090   939.310   939.300    -0.010

XP02              378886.020  5374045.000   185.420   185.420    +0.000

Average dz           +0.003

Minimum dz           -0.070

Maximum dz           +0.100

Average magnitude     0.032

Root mean square      0.039

Std deviation         0.040
West

Number               Easting     Northing   Known Z   Laser Z        Dz

-----------------------------------------------------------------------

XP1_2014          364173.812  5356846.024   139.017   138.990    -0.027

XP2_2014          361739.403  5357174.379   390.814   390.810    -0.004

XP2A_2014         362387.894  5357616.455   191.391   191.400    +0.009

XP3_2014          361837.498  5360004.146   210.648   210.680    +0.032

XP4_2014          363920.881  5361533.324   163.460   163.440    -0.020

Average dz           -0.002

Minimum dz           -0.027

Maximum dz           +0.032

Average magnitude     0.018

Root mean square      0.021

Std deviation         0.024




	Spatial Accuracy (Hz) metres 
	0.4m at 1SD 

	Spatial Accuracy (Vt) metres 
	0.20m at 1SD 

	Surface Type 
	Various 

	Data Tile size (km²) 
	1km2 

	Processing / Derivation Lineage 
	Ground Classifications

Fugro Spatial Solutions Pty Ltd aim is to select those points that are part of a clean ground surface, which might not be 100% of the ground hits. This results in a cleaner dataset, without compromising on terrain detail and accuracy. Ground filtering algorithms, tailored for this project, data type and terrain type, are applied to the full dataset. The ground filtered dataset is then visually checked by an operator, and incorrectly classified data is corrected or the ground filtering algorithm is adjusted and then visually checked. For the verification of the ground classifications, intensity imagery is used as a backdrop image. 

Ground Surface DEM

This dataset consists of every point that is part of the classified ground surface. 
Gridded Ground DEM
A 2m gridded DEM is generated from the ground surface. The height for each grid point is by triangulation between the three nearest DEM ground points.
Model Keypoints

A resampled ground dataset, consisting of only those points that actually contribute to the shape of the terrain (+/- 15cm), with a minimum point density of 1 point every 10 m.

Contours

Triangles are built from the model keypoint DEM. Height points at the required interval are interpolated on the triangle lines and these heights points connected with smooth splines. The splines for each height are then merged to become final contours.

The tiled data has been cut into 1km tiles, where the file name prefix corresponds with the Lower Left coordinate of the block. 



	Limitations of Data 
	All data achieved specification 

	Conditions of Supply 
	1. This Metadata file is always to be stored with the unaltered data contained in this volume. 

2. Any supplied data is to be used only for the purpose for which it was commissioned and in accordance with the terms of engagement. 

3. Use of any supplied data for purposes other than those stated is entirely at the risk of the Client.



Metadata


_123242788.doc
[image: image1.png]-Iimnn







