Laboratory Report

LJN2020-038

# Mineralogical / asbestos analyses, Brassey South

# Mineralogical / asbestos analyses, Brassey South

An unpublished Mineral Resources Tasmania Report

For S. Newett, MRT

By:      	R S Bottrill

Date:   	22/04/2021

By:      	R.S. Bottrill and L Unwin

Date:   22 April 2021

# Summary

Drillcore from Brassey South, Heazlewood, was sampled and analysed to check for potential asbestos. Petrological and XRD investigation revealed the fibrous veins contained mostly hydromagnesite with lizardite and trace pyroaurite and brucite with very little or no asbestiform component.

# Introduction

# Drillcore from Brassey South (ID #92115) drillhole near Heazlewood, NW Tasmania, appeared to contain abundant veins of fibrous white material, potentially asbestos, so it was sampled and analysed to mineralogically check for asbestos, with its details shown in Table 1 & 2.

Table 1: Sample Details

| Reg. No | Drillhole details | Location | Process | Description |
| --- | --- | --- | --- | --- |
| C113515 | BS001 (ID#92115), 14.8m | Brassey | XRD, PL Microscopy | White fibrous veins in serpentine |

# Sample description

# The sample is mostly mid to dark green massive serpentine with abundant friable and roughly fibrous white veins, from <1 mm to a few cm thick (Fig. 1). No asbestiform fibre could be seen.

Figure 1, C113515. White fibrous veins in serpentine. FOV ~120mm.

Figure 2. C113515. White fibrous veins in serpentine. FOV ~120mm.

# XRD analyses

The samples were prepared, examined and analysed in the MRT laboratories, Rosny Park, Tasmania. They were run on a Rigaku Miniflex 600 X-Ray Diffractometer system: a 600W generator 150mm goniometer with a Cu tube; 40kV/15mA, sample spinner and a Scintillation counter (SC) with Be window, -3o to 145o 2Ø scanning range and 3o - 145o 2Ø measuring range, with a scanning speed of 0.01 to 100o/min, a graphite counter monochromator and a Kß Ni- filter.  The analysis software used is the PDXL2 using the ICCD database.

Quantification is largely manual, using a series of prepared standards of the more common minerals to enable some semi-quantitative analysis. Quartz, if present, is used as an internal standard; and if not present, it is often added to the sample for a supplementary scan.  Our semi-quantitative results are calculated using single-peak calibration factors derived from scans of known mixtures of minerals.

The results are shown in Appendix 1 and Table 1.

Table 1: Sample Details

| Phase name | Approx. content |
| --- | --- |
| Hydromagnesite | Dominant |
| Lizardite | Sub-major |
| Pyroaurite | Trace |
| Brucite | Trace |

# Fibre examination

# The samples were prepared and examined by low to high power plain and polarised light microscopy in the MRT laboratories, Rosny Park, Tasmania. This examination revealed that it contains some coarse fibre but very little or no asbestiform fibrous material.

Table 2: Microscopic Examination

| Reg. No. | Location | Observation |
| --- | --- | --- |
| C113515 | Brassey South | No asbestos fibres |

Conclusions and Discussion

# There was no asbestiform material found in this sample. The fibrous white veins consist mostly of hydromagnesite with trace amounts of pyroaurite and brucite. The serpentine matrix appears to be mostly lizardite. The risk of asbestos with this material is considered very low.

# R.S. Bottrill 						L Unwin

# Mineralogist/Petrologist 		Technical Officer

Disclaimers

While every care has been taken in the preparation of this report, no warranty is given as to the correctness of the information and no liability is accepted for any statement or opinion or for any error or omission. No reader should act or fail to act on the basis of any material contained herein. Readers should consult professional advisers. As a result the Crown in Right of the State of Tasmania and its employees, contractors and agents expressly disclaim all and any liability (including all liability from or attributable to any negligent or wrongful act or omission) to any persons whatsoever in respect of anything done or omitted to be done by any such person in reliance whether in whole or in part upon any of the material in this report. The MRT laboratories are not NATA registered but work to similar standards.

This and other data collected in MRT laboratories may enter the MRT databases but every attempt will be made to ensure it remains closed file and not be available externally, unless at your request.

# Appendix 1: MRT Laboratory Report

Client:  MRT

Sample Location:  Brassey South

Job Number:  LJN2021-038

Analyses:  Approximate Mineralogy

Methods: XRD

Analyst:  L Unwin

Lab Manager: R Bottrill

Date:  16/04/2021

# XRD Results-C113515

## General Information

| ### Measurement date: | 16/4/2021 | ### Interpretative date: | 16/4/2021 |
| --- | --- | --- | --- |
| ### Job Number/Client: | LJN2021-038 OHS | ### XRD | Rigaku Miniflex 600 |
| ### Registration Number: | C113515 | ### Analyst: | LUnwin |
| ### Quantitative Method: | Semi-quantitative | ### Process Medium: | Selective white material |
| ### Sample Holder: | Low Reflectance Holder | ### Speed (deg/min): | 0.5 |
| ### Comment: | Shifted -0.15 |  |  |

## Analysis Results

| ### Phase name | ### Content - Estimate |
| --- | --- |
| Hydromagnesite | Dominant |
| Lizardite | Sub-major |
| Pyroaurite | Trace |
| Brucite | Trace |

### Notes

Peak overlap may interfere with identifications and quantitative calculations.

Amorphous minerals and minerals present in trace amounts may not be detected.

## Phase Data Pattern