RL3/2006

# King Lyell Copper Clays Resource Evaluation

August 2013

Author:		 Brendan McGee

(Senior Mine Geologist)

…………………….…

CMT report no:

Distribution:

MRT - Hobart

CMT – Queenstown

# Introduction

RL 3/2006 is a 2km2 retention licence held by Copper Mines of Tasmania on the eastern boundary of mining lease 1M/95. The retention licence covers three known copper bearing mineralised deposits including the Lyell Consols, Lyell Blocks and King Lyell deposits. RL3/2006 is located within the land district of Montagu and in the vicinity of Linda (Figure 1).

Figure 1. Location of RL3/2006.

The King Lyell deposit has been assessed as the only deposit of potential tonnage to be considered for future mining (AMC, 2008).

## Geology

The King Lyell deposit is located within highly deformed and weathered limonitic clays and iron hydroxides derived from Gordon Group Limestone and shales. The deposit is located within a Devonian aged syncline and underlain by the Pioneer sandstone which unconformably overlies the Late Cambrian early Ordovician Owen Conglomerate (Godsall, 2005).  The deposit is overlain with up to 60m of glacial, periglacial and anthropogenic material in the east (Figure 2).  The cover shallows towards the west where the host rock crops out.

Figure 2. Surface geology map of the King Lyell area (AJS and Ken Morrison)

## Drilling data used in resource evaluation

Drilling data selected was from three programmes, the 1996 (three RC holes), 2005 (six diamond holes) and 2011 (11 combination rotary mud pre-collars with diamond tails) (Table 1).  All data previous to these programmes has been not considered due to poor recoveries or unreliable surveys.

| Hole ID | EOH Depth | Method | Year |
| --- | --- | --- | --- |
| 96KLC0001 | 70 | Reverse circulation | 1996 |
| 96KLC0002 | 67 | Reverse circulation | 1996 |
| 96KLC0003 | 60 | Reverse circulation | 1996 |
| 05KLD001 | 32.2 | Diamond (HQ3) | 2005 |
| 05KLD002 | 72.2 | Diamond (HQ3) | 2005 |
| 05KLD003 | 42.4 | Diamond (HQ3) | 2005 |
| 05KLD004 | 48.8 | Diamond (HQ3) | 2005 |
| 05KLD005 | 46.6 | Diamond (HQ3) | 2005 |
| 05KLD006 | 93.7 | Diamond (HQ3) | 2005 |
| DD11CMT007 | 90 | Rotary mud pre-collars with Diamond HQ3 tails | 2011 |
| DD11CMT008 | 99.7 | Rotary mud pre-collars with Diamond HQ3 tails | 2011 |
| DD11CMT009 | 87.1 | Rotary mud pre-collars with Diamond HQ3 tails | 2011 |
| DD11CMT010 | 74.4 | Rotary mud pre-collars with Diamond HQ3 tails | 2011 |
| DD11CMT011 | 87.7 | Rotary mud pre-collars with Diamond HQ3 tails | 2011 |
| DD11CMT012 | 82.4 | Rotary mud pre-collars with Diamond HQ3 tails | 2011 |
| DD11CMT013 | 68.5 | Rotary mud pre-collars with Diamond HQ3 tails | 2011 |
| DD11CMT014 | 95.8 | Rotary mud pre-collars with Diamond HQ3 tails | 2011 |
| DD11CMT015 | 59.4 | Rotary mud pre-collars with Diamond HQ3 tails | 2011 |
| DD11CMT016 | 60.7 | Rotary mud pre-collars with Diamond HQ3 tails | 2011 |
| DD11CMT017 | 51.5 | Rotary mud pre-collars with Diamond HQ3 tails | 2011 |

Table 1. Summary of drill holes used in resource evaluation.

## Resource evaluation – solid geology model

### King Lyell Fault

Initially, the sub-vertical King Lyell Fault was extended under the cover to the south east to provide a southern boundary to mineralisation.  Using this extrapolation as a boundary to mineralisation is a reasonable assumption due to absence of mineralisation found in drill holes 96KLC0003, 05KLD004, 05KLD006, DD11CMT016 and DD11CMT017 (south of boundary, Figure 3).

Using this extrapolation as a fault for solid geology modelling is less robust due to lack of intercepts in drill holes and uncertainty in the geometry of the fault where mapped.  Drill intercepts suggest that the Pioneer Beds are increasingly offset to east (south side up) across this zone.  This could be a result of faulting or fold asymmetry.

Figure 3. King Lyell Fault as mapped (grey dashed) and King Lyell Fault extension (red dashed).  1996 programme=green, 2005 programme=red, 2011 programme=blue collars.

### Pioneer Beds

The top of the Pioneer Beds represent the lower surface of the Gordon Group and hence the lower limit of the King Lyell resource.  The upper surface of the Pioneer Beds was modelled using a combination of drill-hole intercepts and surface geology.  The model of the Pioneers beds took into account both the form of the tight Devonian folding and the extension of the King Lyell Fault.  The geometry of the Pioneer Beds is well understood to the north, south and west, however, due to lack of drilling there remains some uncertainty to the east.

The results of the modelling are displayed in Figure 4.  The Pioneer Beds have an overall east-west synclinal form that shallows to the west.  There is a subtle antiformal east-west fold in the centre of the syncline.  The surface is also modelled with a vertical offset of up to 25m at the eastern end of the King Lyell fault extension.

### Copper Clays resource wireframe

The mineralised portion of the copper clays was modelled from both the drill-hole grade and lithology intercepts and the surface geology under the following constraints:

1) The wireframe was to include drill-hole intervals greater than 0.4 % Cu.

2) The wireframe was not to cross to the south of the King Lyell Fault boundary.

3) The wireframe was to remain above the Pioneer Beds surface.

4) The wireframe was to remain below topography.

5) The wireframe was to include surface mapped copper bearing clays.

Figure 4. Modelled Pioneer Beds surface (Pink) and King Lyell Fault extension (grey) looking west.  Drill holes displayed (Pioneer Beds = pink, Gordon Group clays = tan, Undifferentiated clays = dark grey, Alluvium and tailings = light grey, Recent fill = green).

Figure 5. Modelled mineralised wireframe (brown), constrained by Pioneer Beds (pink) and King Lyell Fault extension (grey).

The resulting wireframe representing the King Lyell resource had a maximum dimensions 220m NW-SE, 90m NE-SW and a maximum thickness of ~20m (Figure 5.).  The deposit dips at 30° towards 120° and has antiformal structure, mirroring the subtle antiform of the underlying Pioneer Beds.  In the centre of the antiform, the wireframe sits above the top of the Pioneer Beds.  The volume of the wireframe is 131,166m3.  It is important to note that the eastern end of the wireframe was modelled entirely on grade due to lack of drill-hole data.

## Resource evaluation – Resource estimation

Resource estimation was carried out in the 315_GRL mine grid.

### Block model parameters

|  | Y | X | Z |
| --- | --- | --- | --- |
| Minimum coordinates | 6900 | 3500 | 2300 |
| Maximum coordinates | 7600 | 5000 | 2700 |
| User block size | 10 | 10 | 2 |
| Sub blocking | 5 | 5 | 1 |
| Rotation bearing | 70 |
| Rotation dip | -30 |
| Rotation plunge | 0 |

### Estimation parameters

The inverse distance squared method was used for grade estimation inside the mineralised wireframe.

The search directions were oriented along strike and down dip.

| Parameter | Magnitude |  |  |
| --- | --- | --- | --- |
|  | Pass 1 | Pass 2 | Pass 3 |
| Min number of samples | 4 | 4 | 2 |
| Max number of samples | 30 | 30 | 20 |
| Max search radius | 50 | 100 | 200 |
| Max vertical search distance | 10 | 20 | 100 |
| Search ellipsoid bearing | 160 | 160 | 160 |
| Search ellipsoid plunge | -30 | -30 | -30 |
| Search ellipsoid dip | 0 | 0 | 0 |
| Ellipsoid Major/semi-major | 2 | 2 | 2 |
| Ellipsoid Major/minor | 4 | 4 | 4 |

### Bulk density

Bulk density was estimated using Pass 3 Cu grade search parameters.  As the bulk densities measured for the clays has varied dramatically in past studies (from 1.9 t/m3 to 2.6 t/m3), two tonnage calculations will be made when stating the resource.  The first will be the estimated value by inverse distance squared and the second will be the value AMC assigned (2.2 t/m3) in the 2008 scoping study.

## Results

Pass 3 model is the only model that completely fills the mineralised wireframe with grade (Figure 7).  It is expected that the Pass 3 model be accurate for volume, but slightly overestimate grade due to the extrapolation of high grades in northern most drill-holes.

It is reasonable to assume that Pass 1 represents an inferred resource.

Figure 6. Plan view of results of Pass 1, 2 and 3 search parameters.  Drill-hole collars in white.  Inset - copper grade legend.

The global resources (inside mineralised wireframe, 0.01 % Cu cut-off) are as follows:

|  | Interpolated density model |  |  | AMC density (2.2 t/m3) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Tonnes | Cu % | Cont Cu (t) | Tonnes | Cu % | Cont Cu (t) |
| Pass 1 | 320,134 | 0.82 | 2625 | 230,890 | 0.82 | 1893 |
| Pass 2 | 379,622 | 0.85 | 3226 | 272,250 | 0.84 | 2286 |
| Pass 3 | 402,134 | 0.91 | 3659 | 287,870 | 0.90 | 2590 |

Figure 7. Grade-tonnage plot for Pass 3 search ellipse (Interpolated density values used).

## Comparison with previous estimations

Due to the results of the 2011 drilling programme, the tonnage of the King Lyell Clays deposit has been dramatically reduced (Table 2).

| Year | Author | Model type | Density (t/m3) | Tonnes (Mt) | Grade (Cu%) | Cont Cu (kt) |
| --- | --- | --- | --- | --- | --- | --- |
| 1998 | CMT |  | 2.55 | 1.2 | 1.37 | 16.4 |
| 2008 | AMC | Upside model | 2.20 | 2.9 | 1.07 | 31.0 |
| 2008 | AMC | Conservative model | 2.20 | 2.5 | 0.90 | 22.5 |
| 2013 | CMT | Pass 3 (Upside) model | Interpolated | 0.4 | 0.91 | 3.6 |

Table 2. Comparison of estimations for the King Lyell Copper Clays.

This reduction in tonnage (volume) in the most recent model is due to the change in the volume of the mineralised wireframe.  The change in volume is due to:

1. Previous models have used the assumption that holes without mineralisation have not drilled deep enough and the wireframe has been modelled to pass underneath these holes (Figure 8).  The 2011 drilling of two holes that were both absent of mineralised clay and ended in Pioneer Beds provided closure for the current model on the west and south-west margins of the deposit (315_GRL relative).

2. Previous models have utilised drilling data from 1901 that has performed poorly in validation drilling.   In these models, a large area of mineralisation has been interpreted on the southern, south-eastern and north western margins (315_GRL relative).  Due to the age and questionable validity, these results have not been used in the current modelling/estimation process.

Figure 8.  Comparison of previous wireframe (green) and current mineralised wireframe (blue) (plan view).  Drill collars shown (1996 programme=green, 2005 programme=red, 2011 programme=blue collars).  Note position of the two 2005 and one 1996 drill holes outside the 2013 wireframe.  The AMC modelled wireframe passes below these holes.

## Further work – boundary delineation

It is recognised that due to the scarcity of reliable drilling data in the south-east and north-west (GRL relative), that the deposit it open in these directions.  It is unlikely that drilling in the north-east will greatly increase the volume of the deposit due to the shallowing nature of the footwall anticline.  However, drilling in the south-east may extend the deposit.