FILE NOTE

SUBJECT:	ZS152A assays from 1040m to 1130m – results not available when the final drilling report was written

DATE:	9 August 2023

A secondary batch of 95 1m samples is referred to in the text of the report.  95 samples were processed initially but these were composited into 2m samples and these are the assay results attached here.  A Stellar Resources geologist provided the following observations.

The additional assays were taken between 1040.00m to 1130.00m, in an effort to try to explain the increase in measured magnetic susceptibility that was found during core logging.  It doesn’t look like these assays have explained this anomaly, with the iron in particular not elevated (which was the prevailing theory to explain the elevated mag sus).

Having said that, there may be an issue with the assay method, with ME-MS61 method utilised as a 4 acid digest.  It is thought that if the iron was as magnetite, this may not have fully dissolved in the acid digest for this method, and thus is the reason low iron numbers were reported.

Stellar think they may need to re-assay the pulps via a different method (XRF) sometime in the future to get a better understanding of the iron content. This work is not currently deemed a high priority and may not be undertaken for some time.

There was an elevated zone of copper noted, 6m at 2315ppm Cu from 1044.00m downhole, but no other significant intercepts have been found.

Steve Richardson

9-8-2023